Split Brain Condition:-

If all the cluster interconnect links fail, it is possible for one cluster to separate

Into two sub clusters, each of which does not know about the other sub cluster.

The two sub clusters could each carry out recovery actions for the departed

Systems. This is termed split brain.

How IO Fencing helps to handle Split Brain Condition:-

When LLT on a system no longer receives heartbeat messages from a system on any of the configured LLT interfaces for a predefined time, LLT informs GAB of the heartbeat loss from that specific system. This predefined time is 16 seconds by default. LLT informs GAB of a heartbeat loss, this happens during a time period known as GAB Stable Timeout (5 seconds). Minimum number of disks required to configure I/O fencing is 3.

GAB marks the system as DOWN, excludes the system from the cluster membership, and delivers the membership change to the fencing module. The system with the lowest LLT system ID in the cluster races for control of the coordinator disks This system races on behalf of all the other systems in its sub cluster. Control is won by the system that gains control of a majority of the coordinator disks.

Coordinator disks are a number of special purpose disks that act together as a global lock device. Racing for control of these disks is used to determine cluster membership and coordinator disks should be SCSI-3 persistent. SCSI-3 Persistent Reservation (SCSI-3 PR) supports device access from multiple systems, or from multiple paths from a single system. At the same time it blocks access to the device from other systems, or other paths). The coordinator disks are placed in a disk group. This disk group is never imported, and

is not used for any other purpose.

The race consists of executing a removal of key of another down host or each key of each system that appears to no longer be in the GAB membership. The abort command allows only a registered system with a valid key to eject the key of another system. This ensures that even when multiple systems attempt to eject other, each race will have only one winner. On the system that wins the race, the vxfen module informs all the systems that it was racing on behalf of that it won the race, and that sub cluster is still valid. This information is passed back to GAB. The other systems in this sub cluster will note the panic, Determine they lost control of the coordinator disks, and also panic and restart.

HAD carries out any associated policy or recovery actions based on the membership changes.

Upon restart, the systems will attempt to seed into the cluster.

If the systems that restart can exchange heartbeat with the number of cluster systems declared in /etc/gabtab, they will

Automatically seed and continue to join the cluster.

In Short : Enabling fencing involves editing the UseFence attribute in the VCS configuration file (main.cf), verifying the configuration file syntax, copying the main.cf to other nodes, setting the contents of the vxfenmode file (DMP or raw), and restarting the fencing driver and VCS.

Each system in the cluster runs a kernel module called vxfen, performs the following actions:

- Registers with the coordinator disks during normal operation.

- Races for control of the coordinator disks during membership changes.

Save the existing configuration:

# haconf -dump -makero

Make a backup copy of the main.cf file:

# cd /etc/VRTSvcs/conf/config

# cp main.cf main.orig

To initialize the disks as VxVM disks, use one of the following methods:

Add three disk and create a DG called "fendg"

# vxdisksetup -i disk_6

# vxdisksetup -i disk_7

# vxdisksetup -i disk_8

# vxdg -o coordinator=on init fendg fendisk01=disk_6 fendisk02=disk_7 fendisk03=disk_8

To check for SCSI3-PR use the command.

# /opt/VRTSvcs/vxfen/bin/vxfentsthdw -g fendg

Deport the coordinator disk group and import the disk group with the -t option to avoid automatically importing it when the nodes restart:

# vxdg deport fendg

# vxdg -t import fendg

Finally we will deport it.

# vxdg deport fendg

Create the file of vxfendg with dg name.

# echo "fendg" > /etc/vxfendg

On all cluster nodes on the SCSI-3 mechanism, type one of the following selections:

# cp /etc/vxfen.d/*dmp /etc/vxfenmode

To check the updated

# more /etc/vxfenmode

Modify two parameter as given below... (Modify on both node)

# vi /etc/sysconfig/vxfen

VXFEN_START=1   (Change it to 1 from 0)

VXFEN_STOP=1 	(Change it to 1 from 0)

Stop VCS on all nodes:

# hastop -all

If the I/O fencing driver vxfen is already running, stop the I/O fencing driver.

# /etc/init.d/vxfen.rc stop

On one node, use vi or another text editor to edit the main.cf file. To modify the list of cluster attributes, add the UseFence attribute and assign its value as SCSI3.

cluster clus1(

UserNames = { admin = "cDRpdxPmHpzS." }

Administrators = { admin }

HacliUserLevel = COMMANDROOT

CounterInterval = 5

UseFence = SCSI3

)

Save and close the file.

# grep SCSI3 /etc/VRTSvcs/conf/config/main.cf

UseFence = SCSI3

Verify the syntax of the file /etc/VRTSvcs/conf/config/main.cf:

# hacf -verify /etc/VRTSvcs/conf/config

Copy the VCS configuration file from this to remaining cluster nodes.

Start the I/O fencing driver and VCS. Perform the following steps on each node:

# /etc/init.d/vxfen.rc start

The vxfen startup script also invokes the vxfenconfig command, which configures the vxfen driver to start and use the coordination points that are listed in /etc/vxfentab

# /opt/VRTS/bin/hastart

=======================================

Verifying I/O fencing configuration

Verify from the vxfenadm output that the SCSI-3 disk policy reflects the configuration in the /etc/vxfenmode file.

On one of the nodes, type:

# vxfenadm -d

Output similar to the following appears if the fencing mode is SCSI3 and the SCSI3 disk policy is dmp:

I/O Fencing Cluster Information:

Fencing Protocol Version: 201

Fencing Mode: SCSI3

Fencing SCSI3 Disk Policy: dmp

Cluster Members:

* 0 (galaxy)

1 (nebula)

RFSM State Information:

node 0 in state 8 (running)

node 1 in state 8 (running)

Check GAB port “b” status

# gabconfig -a

GAB Port Memberships

==================================

Port a gen 24ec03 membership 01

Port b gen 24ec06 membership 01

Port h gen 24ec09 membership 01